| CUSTOMER: AF6182 | ORDER QTY: 20 PCS. | # TOPNOTCH QUALITY WORKS # SHOP TRAVELER | ### APPROVED BY: ### Eddy Chainamnaris | ### RELEASE ### DATE: |
| --- | --- | --- | --- | --- |
| PART NAME: NUT LOCK |  |  | RISK: GREEN |  |
| MATERIAL: CRES 300 SERIES PER QQ-S-763, QQ-S-764 |  |  |  |  |
| NOTE: |  |  |  |  |

| OP. # | OPERATION DESCRIPTION JOB# L16492 | QTY. RECEIVE | OPER. INITIAL | DESCRIPTIONS/ SUPPLIER/ PO#/ CERTIFICATION#/ HEAT#/ PROGRAM# | QTY. ACCEPT | QTY. REJECT | QA STAMP & DATE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M1 | MATERIAL SIZE: Ø13/16 OR BIGGER OR HEX 11/16 TOTAL LENGTH: ______________________ |  |  | SUPPLIER: PO# HT# |  |  |  |
| M2 | MATERIAL SIZE: ______________________ TOTAL LENGTH: ______________________ |  |  | SUPPLIER: PO# HT# |  |  |  |
| 010 | LATHE OP #1 -FACE CUT AND TURN O.D. TO Ø0.812. -DRILL, BORE AND I.D. THREAD TO 0.4372-32 UN-2B. -CUT OFF 0.130-0.140. -QA TO APPROVE AND STAMP FA 1 PC ON THE INSPECTION SHEET FOR OP #010. -INSPECT AND RECORD PER AQL 1% OR 1 IN EVERY 10 PCS. |  |  |  |  |  |  |
| 020 | MILLING OP #2 -CLAMP PART ON FIXTURE. -DRILL 3X Ø0.045-0.050. -CUT HEX 11/16 (0.687 ±0.005). -QA TO APPROVE AND STAMP FA 1 PC ON THE INSPECTION SHEET FOR OP #020. -INSPECT AND RECORD PER AQL 1% OR 1 IN EVERY 10 PCS. |  |  |  |  |  |  |
| 030 | DEBURR PROCESSING -BREAK SHARP EDGES 0.005-0.015. |  |  |  |  |  |  |
| 040 | OUTSIDE PROCESSING -PASSIVATE PER MIL-S-5002. |  |  |  |  |  |  |
| 050 | FINAL INSPECTION QA TO PERFORM C=0, AQL=1.0%, LEVEL II, SINGLE SAMPLING INSPECTION PER WI# 7.4.3. |  |  |  |  |  |  |
| 060 | PUT PART IN EGG CRATE AND BAG WITH TAG INDIVIDUALLY. WATCH FOR FOD. PUT IN THE BOX & APPLY LABEL ON THE BOX. |  |  |  |  |  |  |
| 070 | PREPARE SHIPPING DOC AND SHIP PER P.O. PREPARE PACKING DOC. AND SHIP TO CUSTOMER PER CUSTOMER INSTRUCTIONS. WATCH FOR FOD. |  |  |  |  |  |  |
| 080 | PERFORM "CONFIGURATION AUDIT" FOR PART NUMBER, REV, COUNT INTEGRITY, OPERATION SIGN-OFF AND RELATED CONFIGURATION ITEMS. |  |  |  |  |  |  |
| REACTION PLAN: 1) 100% INSPECTION SINCE LAST OPERATOR CHECK Comments: |  |  |  |  | QTY. TO SHIP: __________PCS. QTY. TO STOCK: _________PCS. |  |  |